# Rescue Drone System

Authentication Specification for Autonomous Rescue Drone System

1. Introduction

This document specifies the authentication framework for the autonomous rescue drone system. It ensures that all commands, mission data updates, and access to sensitive telemetry are cryptographically verified, preventing unauthorized control and protecting the integrity of rescue operations. The system employs a public key infrastructure (PKI) with hardware security modules to achieve robust, scalable authentication.

2. Authentication Architecture Overview

The authentication strategy is based on a PKI model with mutual authentication between the Ground Control Station (GCS) and the drone fleet. This ensures that both ends of the communication link verify each other's identity before any data exchange.

Ground Control Station (GCS): Acts as the command authority. It possesses a private key for signing commands.

Drone: Possesses a unique public key certificate. It verifies the signature on all incoming commands from the GCS.

Mutual TLS (mTLS): All communication channels utilize TLS 1.3 with mutual authentication, requiring both the GCS and the drone to present and validate X.509 certificates.

3. Cryptographic Command Signing & Verification

This is the primary defense against remote command injection attacks (T1 from Threat Model).

3.1. Command Format

Every command sent from the GCS to the drone must be a structured, signed JSON object.

{

"command_id": "cmd_20251005_143022_001",

"mission_id": "rescue_tunisia_earthquake_001",

"timestamp": 1738687422,

"nonce": "a3f59d8c7b2e1f4a6c8d9e0f1a2b3c4d",

"sequence_num": 42,

"command_type": "goto_waypoint",

"parameters": {

"lat": 36.8065,

"lon": 10.1815,

"alt": 50

},

"signature": "3045022100f3a2...b8c4"

}

Table: Signed Command Structure

| Field | Description | Security Purpose |
| --- | --- | --- |
| Command_id | Unique identifier for the command. | Logging, non-repudiation. |
| nonce | Cryptographically secure random number (32-byte). | Replay attack protection |
| sequence_num | Monotonically increasing number. | Replay and out-of-order command protection. |
| signature | Digital signature of all other fields. | Authenticity and integrity verification. |

3.2. Verification Process on Drone

Upon receipt, the drone performs the following steps:

Parse the incoming message.

Extract Signature: Isolate the signature field from the concatenated message fields.

Compute Hash: Calculate SHA-256(concatenated_fields).

Verify Signature: Perform ECDSA_Verify(H, signature, public_key_ground_station).

Check Nonce & Sequence: Validate the nonce has not been used recently and the sequence number is higher than the last accepted command.

Execute or Reject: If all checks pass, execute the command. If any fail, reject the command and log a security event.

4. Mutual Authentication & Secure Key Lifecycle

4.1. Mutual Certificate-Based Authentication

All communication channels (Satellite, LTE) use TLS 1.3 with mutual authentication .

Drone and GCS Certificates: Both parties present X.509 certificates signed by a private, internal Certificate Authority (CA) dedicated to the rescue operation.

Certificate Validation: Validation includes checks for CA signature, expiration, revocation status (via CRL/OCSP), and expected Common Name.

4.2. Secure Key Storage & Lifecycle

Hardware Security Modules (HSMs): All private keys (for GCS signing and drone certificates) are stored in tamper-resistant HSMs (e.g., YubiKey for GCS, Microchip ATECC608A on the drone). Private keys are never exposed to the application OS .

Key Rotation:

Session Keys: Derived fresh for each TLS session (Perfect Forward Secrecy).

Device Certificates: Rotated annually.

CA Root Key: Rotated every 5 years via an offline ceremony.

Revocation: A Certificate Revocation List (CRL) is maintained and distributed. Drones check the CRL before accepting connections from a GCS.

5. Multi-Factor Authentication for Critical Operations

For high-risk commands that could compromise the mission or drone safety, a multi-signature scheme is required, aligning with the principle of requiring multiple independent keys for critical operations .

Commands requiring MFA:

format_storage

disable_safety_limits

modify_geofence

firmware_update

Process:

Operator Signature: The primary operator signs the command using their HSM.

Security Officer Signature: A separate security officer independently signs the same command payload using a different HSM.

Verification: The drone verifies both signatures before execution.