Rescue Drone System

Encryption Specification for Autonomous Rescue Drone System

1. Introduction

This document defines the encryption standards for the autonomous rescue drone system. The primary objective is to ensure end-to-end confidentiality and integrity of all data in transit between the drone and the Ground Control Station (GCS), protecting against eavesdropping and tampering, which are critical for mission success and victim privacy.

2. Encryption Architecture & Standards

The system employs a layered encryption approach, securing both the command channel and the data links.

Protocol: TLS 1.3

Cipher Suite: TLS_CHACHA20_POLY1305_SHA256

Key Exchange: X25519 (Elliptic Curve Diffie-Hellman over Curve25519)

Rationale for Cipher Suite: ChaCha20 is a stream cipher known for high performance on embedded ARM processors (like those found in drones), and Poly1305 provides strong message authentication. This combination is efficient and secure for resource-constrained devices .

3. End-to-End Encrypted Communication

3.1. TLS 1.3 Handshake with Mutual Authentication

The connection establishment follows a standard TLS 1.3 handshake, modified for mutual authentication.

ClientHello (Drone → GCS): Drone sends supported ciphers and its ephemeral public key.

ServerHello (GCS → Drone): GCS selects cipher suite, sends its ephemeral public key, and its certificate.

Certificate Verification (Drone → GCS): Drone sends its certificate and a signature verifying the handshake.

Key Derivation: Both parties use ECDH with their private keys and the other's public key to compute a shared secret.

Session Keys: Session encryption keys are derived using HKDF from the shared secret.

This process provides Perfect Forward Secrecy (PFS), ensuring that a compromise of the long-term private keys does not allow decryption of past recorded sessions .

3.2. Encrypted Message Format

Once the session is established, all messages are encrypted with the following structure:

Nonce: A unique 96-bit value (never reused with the same session key).

Ciphertext: ChaCha20_Encrypt(plaintext, session_key, nonce)

Authentication Tag: Poly1305_MAC(ciphertext, session_key)

This format guarantees confidentiality (ciphertext) and integrity (authentication tag).

4. Data Classification & Encryption Mapping

Different types of data have varying sensitivity levels and are protected accordingly.

| Data Type | Sensitivity | Encryption |
| --- | --- | --- |
| Commands & Mission Updates | CRITICAL (Integrity) | TLS 1.3 + Cryptographic Signing |
| Real-Time Video Feeds | HIGH (Confidentiality) | TLS 1.3 (All streams) |
| Telemetry (Position, Health) | HIGH | TLS 1.3 |
| Victim Detection Data | HIGH (Privacy Sensitivity) | TLS 1.3 |

5. Key Management

Session Keys: Ephemeral, derived during TLS handshake and destroyed after session termination.

Long-term Private Keys: Stored in Hardware Security Modules (HSMs) as specified in the Authentication Spec. Never exposed to system memory.

Public Key Certificates: Stored in drone firmware, verified at boot via secure boot process.