# Rescue Drone System

Firmware Security Specification

1. Introduction

This document outlines the security measures to protect the drone's firmware and software from tampering, unauthorized modification, and execution of malicious code. Ensuring firmware integrity is critical, as a compromise could lead to a persistent backdoor, total system failure, or malicious behavior during rescue operations.

2. Secure Boot & Chain of Trust

A cryptographically verified chain of trust ensures that only authorized software is executed on the drone from power-on.

Immutable Bootloader: The initial bootloader is stored in write-protected flash or ROM. It contains a public key.

Verification Process:

On power-up, the bootloader verifies the digital signature of the next-stage bootloader or operating system kernel using its embedded public key.

This process repeats for each subsequent stage (e.g., kernel verifies root filesystem, drone application).

Any invalid signature halts the boot process, leaving the drone in a safe state.

This process prevents the execution of any unsigned or tampered firmware .

3. Signed Firmware Updates

All firmware updates, whether for the flight controller, companion computer, or radio modules, are delivered as signed packages to prevent supply-chain attacks and malicious updates.

Update Package Format: The update is a compressed archive containing the firmware image, a manifest file with version information, and a cryptographic signature.

Verification on Drone: Before applying an update, the drone:

Verifies the signature of the update package using the vendor's public key (stored in a secure element).

Checks the version number against the current firmware to enforce rollback protection.

Only proceeds with the update if all checks pass.

4. Runtime Integrity Monitoring

To detect runtime tampering, the system employs continuous integrity checks.

Measured Boot: The boot process measures (hashes) each component as it loads and extends these measurements into a secure element (TPM).

Remote Attestation: The GCS can challenge the drone to report its current measurements. The drone responds with the hashes signed by the TPM, allowing the GCS to verify its software state.

File Integrity Monitoring (FIM): Critical system files and configurations are monitored for changes using checksums.

5. Hardware Root of Trust

The security measures above rely on a Hardware Root of Trust.

Secure Element: A chip like the Microchip ATECC608A is used to:

Securely store the public keys for secure boot and firmware verification.

Perform cryptographic operations in isolation.

Provide a unique device identity.

Trusted Platform Module (TPM): Used for storing runtime integrity measurements and key generation.