# Rescue Drone System

Redundancy and Failover Specification

1. Introduction

This document specifies the redundancy and failover strategies for the drone's communication and navigation systems. The primary goal is to ensure high availability and operational continuity even in the face of targeted attacks (like jamming) or component failures, which is paramount in life-critical rescue missions.

2. Multi-Band Communication Failover

To mitigate RF Jamming (T5), the drone is equipped with multiple, diverse communication systems. An intelligent link manager continuously monitors all channels and automatically fails over to the best available one .

2.1. Communication Stack

Primary Link (High Bandwidth): 2.4 GHz WiFi for video streaming and complex data.

Secondary Link (Long Range): 900 MHz LoRa for basic commands and telemetry.

Tertiary Link (Alternative High Bandwidth): 5.8 GHz WiFi.

2.2. Failover Protocol

Continuous Monitoring: The link manager monitors Signal-to-Noise Ratio (SNR), packet loss, and latency on all channels.

Detection: If the primary channel degrades (packet loss >30% for 3 seconds), a jamming event is detected.

Switch: The system automatically switches to the next-best available channel.

Notification: The GCS is alerted: "Jamming detected on 2.4GHz, switched to 900MHz LoRa."

Escalation: If all terrestrial links are lost, the system attempts to establish a satellite connection.

3. Redundant Navigation Systems

To mitigate GNSS Spoofing (T6) and failure, the drone employs sensor fusion and alternative positioning methods.

Primary: Multi-constellation GNSS (GPS, Galileo, GLONASS).

Secondary: Inertial Navigation System (INS) fused with Visual Odometry (VO).

Failover Behavior: If GNSS spoofing is detected (discrepancy between GNSS and INS/VO position), the drone switches to INS/VO-based navigation and alerts the operator.

4. Mesh Networking (Drone-to-Drone Relay)

For swarm operations, a LoRa-based mesh network provides communication redundancy.

Protocol: AODV (Ad-hoc On-Demand Distance Vector) or BATMAN.

Benefit: If a drone's direct link to the GCS is jammed, it can relay its data through other drones in the mesh, creating a resilient, self-healing network.

5. Autonomous Fail-Safe Behaviors

When all communication is lost, the drone enters a pre-programmed, intelligent autonomous mode.

If battery >50%: Continue the current mission segment (e.g., complete the search pattern), buffering all data locally.

If battery <50%: Execute a Return-to-Home procedure via a pre-planned safe route.

On reconnection: The drone immediately transmits all buffered data to the GCS.