# Rescue Drone System

Anomaly Detection Specification

1. Introduction

This document details the anomaly detection system designed to identify unusual patterns in drone behavior, telemetry, and commands. This serves as an advanced line of defense, potentially detecting novel attacks or system faults that other security measures might miss.

2. Architectural Overview

The system employs a two-tiered approach:

On-Drone Real-Time Detection: Lightweight models for immediate reaction to critical anomalies (e.g., command flooding).

Ground-Based Deep Analysis: More complex models on the GCS for analyzing aggregated telemetry and video data to identify subtle or complex threats.

3. Telemetry & Behavioral Anomaly Detection

Using unsupervised learning models like Isolation Forest or Local Outlier Factor (LOF), the system establishes a baseline of normal drone behavior .

Features Monitored:

Flight dynamics (unexpected acceleration, attitude)

Power consumption profiles

Sensor reading correlations (e.g., IMU vs. visual odometry)

Command rate and type frequency

A significant deviation from the baseline, indicated by a high anomaly score, triggers a security alert .

4. Command & Control Anomaly Detection

This layer analyzes the stream of commands for suspicious patterns that might indicate a compromised GCS or an attacker testing the system.

Detection Rules:

Rate Limiting: Enforce a maximum of 10 commands per second from any source.

Impossible Commands: Flag commands that are physically impossible (e.g., instant extreme velocity change).

Geofence Violation Attempts: Detect repeated commands to fly outside the mission's geofence.

Sequence Analysis: Identify illogical command sequences that deviate from standard mission protocols.

5. AI Model for Victim Detection Anomalies

The primary AI model for victim detection is also monitored for anomalies.

Contextual Inconsistency: Flagging detections that are highly improbable for the environment (e.g., a "person" detected in the middle of a lake).

Performance Drift: Monitoring the model's confidence scores for significant drops, which could indicate adversarial attacks or environmental conditions the model wasn't trained for.

6. Response to Detected Anomalies

The response is proportional to the anomaly's severity and confidence.

Low Severity: Log the event for later analysis by security personnel.

Medium Severity: Alert the operator with a request for confirmation.

High Severity: Trigger an immediate autonomous fail-safe action, such as hovering, aborting the current task, or initiating a return-to-home procedure.