# Rescue Drone System

Anomaly Detection Specification

1. Introduction

This document describes the measures to protect the drone from spoofing attacks, where an attacker forges signals to deceive the system. The two primary spoofing threats addressed are GNSS (GPS) Spoofing and Communication/Sensor Spoofing.

2. GNSS Anti-Spoofing Measures

A multi-layered approach is used to ensure the drone cannot be tricked by fake satellite signals.

2.1. Multi-Constellation & Multi-Frequency Receivers

The drone uses a GNSS receiver (e.g., u-blox ZED-F9P) that can simultaneously process signals from GPS (USA), Galileo (EU), GLONASS (Russia), and BeiDou (China).

It also uses multiple frequency bands (L1, L2, L5). An attacker must spoof all constellations and frequencies simultaneously, which is highly complex and requires expensive equipment .

2.2. Signal Authentication

Galileo OSNMA: The drone prioritizes Galileo signals when possible to utilize the Open Service Navigation Message Authentication (OSNMA) feature, which cryptographically authenticates the navigation data, making it impossible to generate valid fake signals without secret keys.

2.3. Cross-Checking with Independent Sensors

The GNSS position is continuously cross-verified with other sensors:

Inertial Navigation System (INS): Uses an IMU for dead reckoning.

Visual Odometry (VO): Uses camera data to estimate movement relative to the ground.

Spoofing Detection: A significant and sustained discrepancy between the GNSS position and the INS/VO position indicates a likely spoofing attack.

3. Communication & Sensor Anti-Spoofing

Cryptographic Authentication: All commands and critical telemetry are cryptographically signed as defined in the Authentication Spec, preventing an attacker from spoofing legitimate commands .

Sensor Data Validation: Data from non-secure sensors (e.g., public ADS-B signals) is treated as untrusted and is not used for safety-critical decisions without validation.

4. Fail-Safe Behavior on Spoofing Detection

Upon detecting a high-confidence spoofing event:

Immediately alert the GCS: "GNSS SPOOFING DETECTED".

Switch to GNSS-denied navigation mode (INS + Visual Odometry).

If alternative navigation is unreliable, execute a pre-defined safe behavior (hover, return to last known valid point).