# Rescue Drone System

Final Security Architecture

Executive Summary

This document synthesizes the complete security architecture for the autonomous rescue drone system, as detailed in the eight constituent specifications. The architecture is built on a defense-in-depth strategy, employing multiple, overlapping security layers to protect the system's critical assets—Flight Control, Mission Data, and Victim Privacy—even under active attack. The design directly addresses the highest-priority threats identified in the Threat Model, including remote command injection, GNSS spoofing, and firmware tampering.

1. Cohesive Security Framework

The architecture is not a collection of independent controls but an integrated framework where each component reinforces the others:

Cryptographic Core (Auth & Encryption): The use of HSM-stored keys for command signing and TLS 1.3 for communication forms the bedrock of trust for all data in transit.

Platform Integrity (Firmware & Physical): Secure boot and tamper detection ensure that the cryptographic core and the rest of the drone's software are running on a trusted hardware platform.

Resilience & Awareness (Redundancy, Anomaly Detection, Anti-Spoofing): These layers ensure the system remains operational and can detect/respond to attacks that bypass the primary cryptographic and integrity controls

2. Defense-in-Depth Against Critical Threats

| Critical Threat | Layered Mitigations |
| --- | --- |
| Remote Command Injection | 1. TLS 1.3 encrypted channel. 2. ECDSA-signed commands. 3. Nonce/Sequence replay protection. 4. Multi-signature for critical commands. |
| GNSS Spoofing | 1. Multi-constellation/frequency GNSS. 2. Galileo OSNMA authentication. 3. INS/Visual Odometry cross-verification. 4. Failover to GNSS-denied navigation. |
| RF Jamming | 1. Multi-band communication (LoRa, WiFi, Satellite). 2. Automatic failover based on link quality. 3. Mesh networking. 4. Autonomous operation during blackout. |
| Firmware Tampering | 1. Hardware-rooted secure boot. 2. Signed firmware updates. 3. Runtime integrity monitoring. 4. Tamper detection and secure key storage. |

3. Conclusion

This security architecture presents a robust, resilient, and comprehensive solution for securing autonomous rescue drones. By systematically addressing threats through cryptographic assurance, platform integrity, and operational resilience, it justifies every security decision with a clear rationale tied to risk mitigation. This architecture not only meets the immediate needs of the competition but also provides a strong foundation for the secure deployment of autonomous systems in real-world, critical rescue operations.